MOBILE PILOT IN COCKPIT WHILE ON EXTERNAL GENERATOR POWER:

  - Master Power		ON

  - INMARSAT		OFF

  - Nose Power		OFF		EXRAD

  - N3 			OFF		EXRAD

  - N4			OFF		EXRAD Tx

  - Upper Q-bay		OFF		LIP, AVS Camera & AVS C&C

  - Q1			OFF		LIP

  - Q4			OFF		AVS Camera & AVS C&C

  - Lower Q-Bay		OFF		AMPR

  - Q6			OFF		AMPR

  - Left Wing		OFF		CoSSIR/HIWRAP

  - L3			OFF		HIWRAP

  - L4			OFF		HIWRAP Tx

  - L1			OFF		CoSSIR Power

  - L2			OFF		CoSSIR Scan

  - Right Wing		OFF		CPL/CRS/MVIS Camera

  - R1 			OFF		CPL Power

  - R2			OFF		CPL Laser Tx

  - R3			OFF		CRS Power

  - R4			OFF		CRS Tx

  - R5			OFF		MVIS Camera

BEFORE ENGINE START:

Verify the following lights and switches

- Master Power		ON

- INMARSAT		OFF

- Nose Power		OFF		EXRAD

- N3 			OFF		EXRAD

- N4			OFF		EXRAD Tx

- Upper Q-bay		OFF		LIP, AVS Camera and AVS C&C

- Q1			OFF		LIP

- Q4			OFF		AVS Camera & AVS C&C

- Lower Q-Bay		OFF		AMPR

- Q6			OFF		AMPR

- Left Wing		OFF		CoSSIR/HIWRAP

- L3			OFF		HIWRAP

- L4			OFF		HIWRAP Tx

- L1			OFF		CoSSIR Power

- L2			OFF		CoSSIR Scan

- Right Wing		OFF		CPL/CRS/MVIS Camera

- R1 			OFF		CPL Power

- R2			OFF		CPL Laser Tx

- R3			OFF		CRS Power

- R4			OFF		CRS Tx

- R5			OFF		MVIS Camera

AFTER POWER TRANSFER:

- INMARSAT		ON

- Right Wing		ON		CPL/CRS/MVIS Camera

- R1			ON		CPL Power R1-FL Steady for up to 10 seconds

Verify R1-FL OFF

- R1			OFF

- R3			ON		CRS Power, R3-FL Steady for up to 5 min

- Right HTR		HTR&BLO

- Nose 			ON

- N3			ON		EXRAD PWR On, N3-FL should be off after 2 min

- Upper Q-bay		ON		LIP, AVS Camera and AVS C&C

- Q1			ON		LIP Record On

- Lower Q-Bay		ON		AMPR

- Q6			ON		AMPR PWR On, Q6-FL Steady for ~1 minute

- Left Wing		ON		CoSSIR/HIWRAP

- L3			ON		HIWRAP PWR On, L3-FL Steady for up to 5 min

Please wait 60 seconds

BEFORE TAXI

Verify lights and switches are ON and no FL

N3 ON, Q1 ON, Q6 ON, L3 ON, R3 ON, No FL lights, Right HTR&BLO

BEFORE TAKEOFF

- R1			ON		CPL Power

AFTER TAKEOFF

- L1			ON		CoSSIR Power

ABOVE 10K FT

- R5			ON		MVIS Camera

- Q4			ON		AVS Camera & AVS C&C

ABOVE 60K FT:

- Ensure autopilot is in “Mach hold” setting

- TACAN 			Rec or A-A Rec before turning L2 ON unless req’d for SA

- L2			ON		CoSSIR Scan

- N4			ON		EXRAD Tx

- N5			ON		EXRAD heater/blower

- L4			ON		HIWRAP Tx

- R2			ON		CPL Laser Tx

- R4			ON		CRS Tx, R4-FL for up to 90 sec

NOTE: DURING NIGHT OPERATIONS IN CANADIAN AIRSPACE R2 SHOULD BE OFF

BEFORE DESCENT:

- N4			OFF		EXRAD Tx

- N5			OFF		EXRAD Heater/Blower

- L2			OFF		CoSSIR Scan

Note: L1-FL and/or L2-FL may be on.

- L4			OFF		HIWRAP Tx

- R2			OFF		CPL Laser Tx

- R4			OFF		CRS Tx

- R5			OFF		MVIS Camera

- TACAN 			As required

BEFORE LANDING (20K FT CHECK)

- N3			OFF		EXRAD

- L1			OFF		CoSSIR Power

- L3			OFF		HIWRAP

- R3			OFF		CRS

- Q4			OFF		AVS Camera & AVS C&C

AFTER PARKED:

- Wait 3 minutes for HIWRAP/CRS/EXRAD

- Nose PWR		OFF		EXRAD

- Q1			OFF		LIP

- Upper Q-bay		OFF		LIP, AVS Camera & AVS C&C

- Q6			OFF		AMPR

- Lower Q-Bay		OFF		AMPR

- Left Wing		OFF		CoSSIR/HIWRAP

- R1			OFF		CPL Power

- Right HTR		OFF

- Right Wing		OFF		CPL/CRS/MVIS Camera

- INMARSAT		OFF

- Master PWR		OFF

Failure Checklist(s)

Contact ground prior to initiating the following checklists

# EXRAD:  N3 or N3 & N4 Fail:

Contact EXRAD team prior to running this checklist.

Wait for 2 minutes

- N4			OFF 		(if ON)

- N3			OFF

Wait for 2 minutes

- N3 			ON		EXRAD PWR On

- N3 fail light should be off after 5 minutes

Proceed to Step 5 only if at altitude

- N4			ON

Repeat Fail Sequence 3x.  If fault lights stay on, N4 OFF and leave N3 ON

AMPR:  Q6 Fail:

(Remains on after~5min during start-up)

Wait until at altitude and proceed with During Flight Failure.

(During Flight)

- Q6			OFF		10 seconds

- Q6			ON		Wait 15 min
Repeat 3 times if required.

LIP:  Q1 Fail:

- Q1			OFF
Wait 10 seconds

- Q1			ON
If Fail continues repeat 3 times, then leave ON

- Q1			ON

NOTE: If Upper Q-bay area power is cycled, will cycle power for MVIS computer

CPL:	R2 or R1&R2 Fail:

Verify altitude above FL530

      - R2			OFF

- R2			ON

If Fail continues

- R2			OFF

- R1			OFF

- R1			ON

Wait 2 min (R1FL Steady for up to 10 sec)

- R2			ON

Repeat up to 3 Times

If Fail continues

- R2			OFF

- R1			OFF

CPL:  R1 Fail:

- R2			OFF		(If ON)

- R1			OFF

- R1			ON

Wait 2 min (R1FL Steady for up to 10 sec)

- R2			ON		(If above FL530)

Repeat up to 3 Times

If Fail continues

- R2			OFF

- R1			OFF

If possible, contact CPL team after running this checklist.

CRS:	R3 or R3&R4 Fail (Fail light on for more than 5 min during start-up):

If possible, contact CRS team prior to running this checklist.

- R4			OFF		(If ON)

- R3			OFF
	Wait 3 min

- R3			ON		(R3-FL for up to 5 minutes)

- R4			ON		(If at Altitude)
	Wait 5 min
	If Fail continues repeat 3 times
	If Fail doesn’t clear

- R4			OFF

- R3			ON

### HIWRAP:  L3 Fail or L3/L4 Fail (wait 5 mins after L3 or L4 ON):

If possible, contact HIWRAP team prior to running this checklist.

- L4			OFF		(If ON)

- L3			OFF
Wait 4 minutes

- L3			ON

Proceed to Step 4 only if at altitude

- L4			ON

Wait up to 5 min for FL to extinguish
If Fails continue repeat 3 times

- L4			OFF

- Leave L3		ON

### CoSSIR:  L1 Fail (During power up – L2 still OFF)

If possible, contact CoSSIR team prior to running this checklist.

  - L1			OFF

  - L1			ON 		(wait 2-3 min for fault light to go out)
If fail still continues CoSSIR team will troubleshoot from ground

  - L1			ON

CoSSIR:  L1 and/or L2 Fail During Flight

- L2			OFF

Alert CoSSIR team to troubleshoot from ground

CoSSIR:  Reboot

- L2			OFF

- L1			OFF
Wait ~30seconds

- L1			ON		L1 Fail light for ~2 minutes
Wait (2-3 min) for L1 FL to go out

- L2			ON

If fail still continues

- L2			OFF
CoSSIR team will troubleshoot from ground